TASK-1

Here are 5 common BIOS or POST error messages and the hardware or configuration issues they indicate:

- "CMOS Checksum Error" (or "CMOS Battery Failure")

  - Issue: The motherboard's small coin-cell battery (CR2032) is dead or dying, causing saved BIOS settings and system date/time to reset whenever the PC is unplugged.

- "No Boot Device Found" (or "Disk Boot Failure")

  - Issue: The hard drive or SSD is disconnected, corrupted, or not selected as the primary boot drive in the BIOS settings.

- "CPU Fan Error"

  - Issue: The processor's cooling fan is unplugged from the header, spinning too slowly, or blocked by dust/cables, putting the CPU at risk of overheating.

- "Keyboard Error" (or "No Keyboard Present")

  - Issue: The system cannot detect a connected keyboard, or a key is stuck down during startup.

- "Memory Size Decreased" (or "RAM Fail")

  - Issue: One of the installed RAM sticks is loose, unseated, or faulty, causing the BIOS to detect less total memory than before.

TASK-2

For an AMI BIOS, 3 beeps indicates a Base 64K RAM Failure (a read/write memory error in the first 64KB of system RAM). This means the motherboard cannot properly detect or communicate with the installed memory sticks.

2 Troubleshooting Steps:

- Reseat the RAM Modules:

Turn off and unplug the PC. Unclip and remove the RAM sticks, gently wipe their gold contacts with a pencil eraser or soft cloth to remove oxidation, and press them firmly back into the motherboard slots until the side clips snap shut.

- Test One Stick and Slot at a Time:

If you have multiple RAM sticks, remove all but one stick and try booting the PC. If it still beeps, move that single stick into a different motherboard slot, or try a different stick to identify if a specific RAM module or motherboard slot is faulty.

TASK-3

As an AI, I cannot access a physical computer's hardware directly or enter a live BIOS, but here is an example of a standard BIOS Setup Utility screen:

Identified Details from Example Screen:

- BIOS Manufacturer: American Megatrends, Inc. (AMIBIOS)

- BIOS Version / ID: 07.01.26.00 (Core v02.61)

How to check this on your own PC:

- Restart your PC and repeatedly press the Delete key or F2 as soon as the screen turns on.

- Look at the Main or Information tab at the top of the screen.

- Find the lines labeled BIOS Version and Motherboard / System Manufacturer.

TASK-4

What it means:

This error means the motherboard lost its saved BIOS settings (such as custom boot options, date, and time) due to lost power or memory corruption. To ensure the PC can still turn on, the system automatically reset and restored its factory default settings.

How to resolve it:

- Replace the CMOS Battery: Turn off and unplug the PC. Locate the coin-cell battery (CR2032) on the motherboard, pop it out, and replace it with a fresh 3V battery (a dead battery is the #1 cause of this error).

- Enter BIOS Setup: Turn on the PC and press the prompted key (usually F1, F2, or Del) to enter the BIOS menu.

- Reset Date, Time, and Settings: Set the correct current date and time, and reconfigure any custom settings (like your boot drive order).

- Save and Exit: Press F10 to save the changes and reboot the PC normally.