TASK-1

RAM Reseating is the physical process of removing a memory module (RAM) from its slot on the motherboard and re-inserting it firmly to re-establish a proper electrical connection.

The Problem Defined

Over time, thermal expansion, physical movement, dust, or microscopic surface oxidation can loosen the connection between the memory module's gold contacts and the motherboard slot pins.

Symptoms that indicate RAM needs reseating:

- No Display / Failure to Boot: The computer powers on (fans spin), but the screen remains black.

- Error Beeps or Motherboard LEDs: Repeating beep codes or an lit DRAM diagnostic LED on the motherboard during POST (Power-On Self-Test).

- System Crashes: Random Blue Screen of Death (BSOD) errors with messages like MEMORY_MANAGEMENT or IRQL_NOT_LESS_OR_EQUAL.

- Missing Capacity: Windows Task Manager shows less usable memory than what is physically installed (for example, showing 8 GB usable out of 16 GB).

Step-by-Step Reseating Guide

1.Power Down and Ground:Prevent static shock and short circuits.

- Turn off the system completely and disconnect the main power cable from the wall (remove the battery if servicing an older laptop).

- Hold the power button for 5 seconds to drain residual electrical power from motherboard capacitors.

- Touch an unpainted metal surface on the chassis to discharge personal static electricity.

2.Open Case and Locate RAM:Access the motherboard memory slots.

- For desktops: Remove the side panel screws and slide off the side panel.

- For laptops: Unscrew and remove the bottom panel.

- Locate the long rectangular DIMM slots next to the CPU cooler (desktop) or flat SODIMM slots (laptop).

3.Release Retention Clips and Remove:Disconnect module safely.

- Desktop: Push down on the plastic retaining latches at the ends of the RAM slot (some modern boards only have a latch on one end). The RAM stick will pop up slightly.

- Laptop: Gently push the metal spring clips on both sides outward until the module pops up at a 30-degree angle.

- Pull the module straight out holding it by its edges. Avoid touching the exposed gold contacts.

4.Align and Re-insert:Match key notch and apply firm pressure.

- Blow away any dust from the slot using compressed air.

- Line up the asymmetrical notch on the bottom of the RAM stick with the plastic key inside the slot.

- Push straight down with equal pressure on both ends until the plastic latches snap inward and lock with a distinct click.

5.Reassemble and Test:Verify system recognition.

- Reattach panels, reconnect power cables, and boot the computer.

- Check BIOS/UEFI or open Task Manager (Ctrl + Shift + Esc > Performance > Memory) to confirm all installed RAM is detected.

Common Issues Observed During Reseating

- Incomplete Insertion (Most Common): The module is pushed down, but not hard enough for the latches to auto-click. This causes "No Post" (blank screen). Uneven pressure can leave one side higher than the other.

- Reverse Alignment Attempt: RAM sticks are keyed off-center to fit only one way. Forcing a module upside down will crack the plastic slot guide or break the pins.

- Corroded or Dirty Gold Pins: Dust or oils from fingers on the gold pins can prevent signal transfer even when fully seated. Gently wiping the gold contacts with a soft microfiber cloth or isopropyl alcohol fixes dirty contacts.

- Incorrect Slot Channel Placement: On motherboards with 4 slots and only 2 RAM sticks, putting them in adjacent slots (e.g., slots 1 and 2 instead of 2 and 4) can disable dual-channel performance or cause boot instability.

TASK-2

Diagnosing a faulty Graphics Processing Unit (GPU) requires systematically isolating potential failure points across display outputs, physical hardware connections, software drivers, and hardware load tests.

1.1. Identify Symptoms & Verify Display Connections:Rule out external cable and monitor faults.

- Check symptoms: Note if the PC shows a black screen, screen tearing/lines (artifacting), random game crashes, or spinning fans with no video output.

- Verify cable placement: Confirm the HDMI or DisplayPort cable is plugged directly into the GPU ports on the back of the case, not the motherboard video port near the USB ports.

- Test display hardware: Connect the PC to a different monitor or TV using a spare cable to ensure the monitor itself isn't the problem.

2.2. Check Physical Connections & Reseat GPU:Ensure power delivery and motherboard contact.

- Power off the PC and flip the power supply switch to OFF.

- Check power cables: Verify that all PCIe power connectors (6-pin, 8-pin, or 12VHPWR cables) are clicked tightly into the GPU with no visible gap.

- Reseat the GPU: Unscrew the rear bracket, press the motherboard PCIe slot latch, remove the GPU, and push it back firmly into the top PCIe x16 slot until the latch clicks shut.

3.3. Perform a Clean Driver Reinstallation:Fix corrupted graphics software or driver conflicts.

- Boot Windows into Safe Mode.

- Run Display Driver Uninstaller (DDU) to completely wipe all existing graphics driver files and registry keys.

- Reboot normally and download/install the latest graphics driver directly from the official manufacturer website (Nvidia, AMD, or Intel).

4.4. Cross-Test the GPU Hardware:Isolate the GPU from other PC parts.

- Try a secondary PCIe slot: Move the GPU to another slot on the motherboard to test for a dead PCIe slot.

- Test in another PC: Place your GPU into a second working computer. If the problem continues on the second machine, the GPU hardware itself is faulty.

- Swap GPUs: Install a known working GPU into your PC. If the PC boots normally, your original GPU needs repair or replacement.

5.5. Stress Test & Confirm the Fix:Verify stability under heavy load.

- Launch a GPU benchmarking tool (such as FurMark or 3DMark) or a high-demand game, and run it for 15 to 30 minutes.

- Monitor hardware stats: Check temperatures using tools like MSI Afterburner or HWMonitor to verify temperatures stay under 85°C.

- If the system remains stable with no crashes, flickering, or artifacts under full load, the fix is confirmed.

TASK-3

Simulating a storage disconnect demonstrates how a computer behaves when a drive experiences loose physical connections, power loss, or hardware failure.

Phase 1: Drive Unplugged (Simulated Fault)

When the SATA data cable or NVMe M.2 module is removed from the motherboard and the system is powered on:

If it is the Primary Drive (OS / Boot Drive):

- On-Screen Error: A black screen displays an error message:

Reboot and Select proper Boot device or Insert Boot Media in selected Boot device and press a key

(On modern motherboards, it may show No Bootable Device Found or boot directly into BIOS setup).

- BIOS / UEFI Indicators:

  - Storage Info: The assigned port displays "N/A", "Empty", or "Not Present" (for example, NVMe_1: N/A or SATA6G_1: Empty).

  - Boot Priority List: The drive name disappears entirely from the boot sequence list.

If it is a Secondary Drive (Data / Storage Drive):

- On-Screen Error: Windows boots normally, but the drive letter (e.g., D: or E:) is completely missing from File Explorer.

- Windows Event Viewer: Logs Event ID 157: Disk X has been surprise removed.

Phase 2: Drive Reconnected (Simulated Fix)

When the system is powered off, the cable or M.2 slot is re-seated securely, and the system is restarted:

If it is the Primary Drive:

- On-Screen Result: The computer passes the motherboard splash screen and boots directly into the operating system without errors.

- BIOS / UEFI Indicators:

  - Storage Info: The exact drive model reappears (for example, SATA6G_1: Samsung SSD 870 EVO 500GB or M.2_1: WD_BLACK SN850X 1TB).

  - Boot Priority List: The drive or "Windows Boot Manager" automatically reclaims its spot as the primary boot target.

If it is a Secondary Drive:

- On-Screen Result: Windows recognizes the hardware upon booting. Opening Disk Management shows the disk as "Online" / "Healthy", and all stored files reappear in File Explorer.

Before vs. After Comparison

| Status Indicator | Drive Unplugged (Fault) | Drive Reconnected (Fixed) |
| --- | --- | --- |
| BIOS Detection | Listed as Empty or N/A | Displays Full Drive Model & Size |
| Boot Screen | No Bootable Device Found Error | Boots directly into OS |
| File Explorer | Drive Letter Missing | Drive Letter Appears Normally |
| Disk Management | Drive Not Listed | Status: Healthy (Primary Partition) |

TASK-4

For random PC restarts, you should check the RAM (Memory) first.

Why Check RAM First?

- Instant Kernel Crashes: RAM holds active operating system files and data. When a RAM module has corrupted memory addresses, the system instantly panics and reboots to prevent data corruption.

- Restarts at Idle or Light Load: Unlike a GPU failure (which usually causes crashes during gaming or high graphics load) or Storage issues (which cause freezing, slow file access, or "No Bootable Device" errors), unstable RAM causes sudden, random restarts even while idle on the desktop.

- Overclock Instability: Profile profiles like XMP (Intel) or EXPO (AMD) can become unstable over time or after BIOS updates, leading to power cycles.

Step-by-Step RAM Diagnostic Guide

1.1. Disable XMP / EXPO in BIOS:Rule out RAM overclock instability.

- Restart your PC and press Del or F2 to enter the BIOS/UEFI setup.

- Locate the memory profile setting (XMP, EXPO, or DOCP) and set it to Disabled or Auto (running at base JEDEC speed).

- Save changes and reboot. If the random restarts stop, your RAM profile requires minor voltage tuning or a BIOS update.

2.2. Run Windows Memory Diagnostic:Use Windows built-in memory scan.

- Press Win + R, type mdsched.exe, and press Enter.

- Select Restart now and check for problems.

- The PC will restart into a blue test screen. If the tool reports hardware errors, one or more RAM sticks are faulty.

3.3. Test RAM Sticks Individually:Isolate hardware module and slot failures.

- Power off the PC and unplug the power cable.

- Remove all RAM sticks except one inserted into slot 2 (or slot A2).

- Boot the PC and use it normally. If it remains stable, swap to the second stick to see if the random restarts return on that specific stick.

4.4. Run MemTest86 for Advanced Diagnostics:Perform a deep, multi-pass memory test.

- Download MemTest86 (free version) and write it to a USB drive using their installer.

- Boot from the USB drive and let the test run through 4 full passes.

- If MemTest86 logs even 1 single error, the RAM module is defective and needs replacement.