TASK-1

When a laptop refuses to power on despite being plugged in, the issue usually stems from a power delivery failure, a protective motherboard circuit triggering, or a dead display.

Here is a systematic checklist to isolate the problem:

1.1. Test the Charger and Outlet:Verify the power adapter and wall socket.

- What to do: Plug the charger directly into a different wall outlet (skip power strips). Check if the LED indicator light on the power brick itself turns on (if it has one).

- What you are checking for: A dead wall outlet, a blown fuse in a power strip, or a completely failed AC power adapter brick. If the charger is dead, the laptop is simply out of battery.

2.2. Inspect the Charging Port:Look for physical hardware damage.

- What to do: Unplug the charger and shine a flashlight into the laptop's charging port. Gently wiggle the connector when plugged in to see if it feels excessively loose.

- What you are checking for: Bent pins, broken plastic, or debris inside the port that prevents the charger from making a solid electrical connection with the motherboard.

3.3. Look and Listen for 'Signs of Life':Isolate a screen failure from a power failure.

- What to do: Press the power button and lean in close. Listen for a spinning fan or a spinning hard drive. Look for the power LED on the side or front of the laptop.

- What you are checking for: A dead display. If the fan spins and the LEDs turn on but the screen is completely black, the laptop is powering on, but the screen (or the cable connecting it) has failed.

4.4. Disconnect All External Devices:Check for external short circuits.

- What to do: Unplug everything except the power cord. This includes USB mice, keyboards, flash drives, external monitors, and SD cards. Try turning it on again.

- What you are checking for: A faulty peripheral. A broken USB device can cause a short circuit, which forces the laptop's motherboard into a protective shutdown to prevent fire or electrical damage.

5.5. Perform a Hard Power Drain:Clear residual static charge.

- What to do: Unplug the charger. If your laptop has a removable battery, take it out. Press and hold the power button down firmly for 30 to 60 seconds. Reconnect the charger (leave the battery out if possible) and press the power button once.

- What you are checking for: Static electricity buildup. Sometimes, residual power gets trapped in the motherboard's capacitors and triggers a safety mechanism that refuses to let the system boot. A hard drain clears this out.

TASK-2

1.1. Check Monitor Power and Status Light:Verify power status and standby modes.

- Check if the monitor's power light (LED) is ON, OFF, or AMBER/ORANGE (sleep mode).

- If OFF: Press the monitor's power button and check that the power cord is securely plugged into both the wall and the back of the monitor.

- If AMBER/ORANGE: The monitor has power but isn't receiving a signal from the PC.

2.2. Inspect Video Cable Connections:Confirm the physical display cables and port locations.

- Unplug and firmly re-seat both ends of the HDMI or DisplayPort cable.

- Crucial check for desktop PCs: Make sure the cable is plugged directly into the dedicated graphics card (GPU) ports located lower down on the back of the case, not the motherboard video ports near the USB connectors.

- If you are using adapters or a docking station, temporarily remove them and connect directly.

3.3. Verify the Monitor Input Source:Match the monitor's active channel to the cable type.

- Press the Input or Source button on the monitor (usually on the bottom or back edge).

- Manually cycle through the options and select the port you are using (e.g., set to HDMI 1 or DisplayPort).

4.4. Reset Graphics Driver and Screen Mode:Attempt built-in Windows display recovery shortcuts.

- Press Win + Ctrl + Shift + B on the keyboard. This forces Windows to restart the graphics driver (you should hear a short beep).

- Press Win + P to open the projection menu, press the Down Arrow key once, and hit Enter (this switches display modes in case Windows is trying to send video to a non-existent second screen).

5.5. Cross-Test Hardware and Reseat RAM:Isolate whether the issue is the monitor, cable, or internal PC hardware.

- Test monitor/cable: Connect a different monitor or TV to the PC using a spare cable. If the second screen displays video, the original cable or monitor is faulty.

- Reseat RAM: If the PC fans spin but no monitor receives a signal, turn off the PC, open the side panel, and unclip and re-seat the RAM sticks. Loose RAM is one of the most common causes of a PC turning on without displaying a picture.

TASK-3

To isolate whether an unresponsive wireless mouse is caused by the mouse itself, a faulty USB port, or Windows system settings, follow this 3-step elimination process.

1.1. Isolate the Mouse Hardware:Rule out dead batteries or a powered-off mouse.

- Check power: Flip the mouse over and ensure the power switch is set to ON.

- Inspect optical light/LED: Look for a red/blue light underneath or a power LED indicator on top.

- Replace battery: Insert a fresh battery or plug the mouse in using its charging cable for 10 minutes.

- Verdict: If there are no lights even with a new battery, the mouse itself is defective.

2.2. Isolate the USB Port & Receiver:Rule out port failure or dead wireless dongle.

- Change ports: Unplug the USB receiver dongle and insert it into a different USB port directly on the PC (avoid unpowered USB hubs).

- Cross-test on another PC: Plug the USB receiver and mouse into a second computer or laptop.

- Verdict: If the mouse works in a different port, the original USB port is bad. If the mouse works on a second PC, the issue is isolated to settings on your main PC.

3.3. Isolate System Settings & Drivers:Use keyboard shortcuts to check device status.

- Open Device Manager: Since the keyboard works, press Win + X and select Device Manager using the arrow keys and Enter.

- Check drivers: Navigate to Mice and other pointing devices, expand the list, and look for a yellow exclamation mark or a disabled device icon. Press Shift + F10 (or the menu key) on the mouse driver to select Enable device or Uninstall device (then reboot to let Windows auto-reinstall it).

- Check Bluetooth (if applicable): Press Win + I, navigate to Bluetooth & devices, and verify Bluetooth is toggled ON and the mouse is paired.

- Verdict: If driver updates or re-enabling Bluetooth restores movement, the issue was software-related.

TASK-4

When a gaming PC suddenly shuts down during heavy use, it is usually triggering a thermal trip—a built-in safety feature that cuts power to prevent the processor (CPU) or graphics card (GPU) from literally melting.

Here are the three most common hardware culprits and exactly how to check them:

1.1. Severe Dust Buildup:Blockages trap heat inside the case.

- The Cause: Gaming cafes are high-traffic environments. Dust, hair, and lint get sucked into the PC and form a thick "carpet" over the heatsink fins, completely suffocating the components.

- How to Check: Power down the PC, unplug it, and remove the side glass panel. Shine a flashlight onto the CPU cooler (the big metal block on the motherboard) and the front intake vents. If you cannot see the metal fins clearly through the dust, it needs to be blown out with compressed air.

2.2. Dead or Failing Fans:Mechanical failure prevents active cooling.

- The Cause: A fan motor has burned out, a cable has come loose, or a wire is physically jamming the fan blades from spinning.

- How to Check: Turn the PC on with the side panel removed. Launch a game to put the system under load. Physically look at the CPU cooler fan, the fans on the graphics card, and the case fans. If a fan is completely still, twitching without spinning, or making a loud grinding noise, it needs to be replaced.

3.3. Dried-out Thermal Paste:Loss of heat transfer between the chip and the cooler.

- The Cause: Thermal paste is a conductive gray gel sitting between the CPU and the cooler. Over years of heavy heat cycles, it dries out, turns chalky, and stops transferring heat away from the processor.

- How to Check: If the fans are spinning fast and the PC is completely free of dust, but diagnostic software (like HWMonitor) shows the CPU instantly spiking to 95°C–100°C under load, the paste is dead. You must unscrew the CPU cooler, wipe away the old crusty paste with isopropyl alcohol, and apply a fresh, pea-sized drop of new thermal paste.