TASK-1

| Device | Power Supply Type | Why It Uses This Power Supply |
| --- | --- | --- |
| Desktop Computer | SMPS | Converts AC power into the DC voltages required by computer components efficiently. |
| LED Monitor | SMPS | Provides stable DC power while consuming less electricity and producing less heat. |
| Wi-Fi Router | SMPS | Supplies regulated DC power for continuous and efficient operation. |
| Desktop Computer (with Backup) | UPS | Keeps the computer running during a power outage so work can be saved safely. |
| Wi-Fi Router (connected to UPS) | UPS | Maintains internet connectivity during short power cuts by providing temporary backup power. |

TASK-2

SMPS (Switched Mode Power Supply)

AC Mains (230V AC)

│

▼

┌─────────────────┐

│   AC INPUT      │

│ (Power Socket)  │

└─────────────────┘

│

▼

┌─────────────────┐

│ EMI FILTER      │

│ (Noise Filter)  │

└─────────────────┘

│

▼

┌─────────────────┐

│ RECTIFIER       │

│ (AC → DC)       │

└─────────────────┘

│

▼

┌─────────────────┐

│ FILTER          │

│ CAPACITOR       │

└─────────────────┘

│

▼

┌─────────────────┐

│ HIGH-FREQUENCY  │

│ SWITCHING       │

│ CIRCUIT         │

└─────────────────┘

│

▼

┌─────────────────┐

│ TRANSFORMER     │

│ (Isolation &    │

│ Voltage Change) │

└─────────────────┘

│

▼

┌─────────────────┐

│ RECTIFIER &     │

│ OUTPUT FILTER   │

└─────────────────┘

│

▼

┌─────────────────────────────────────────┐

│ DC OUTPUT                              │

│ +12V   +5V   +3.3V   -12V   +5V Standby │

└─────────────────────────────────────────┘

│

▼

Motherboard, CPU, HDD/SSD, GPU, Fans

TASK-3

| Power Connector | Devices It Powers | Safety Tip |
| --- | --- | --- |
| ATX 24-pin Motherboard Connector | Supplies the main power to the motherboard. | Always turn off the PC and unplug the power cable before connecting or disconnecting it. Press the locking clip before removing it. |
| SATA Power Connector | Powers SATA HDDs, SATA SSDs, and DVD/Blu-ray drives. | Insert it in the correct orientation. Do not force the connector, as it can be damaged. |
| 4-pin Molex Connector | Powers older hard drives, optical drives, cooling fans, and accessories. | Hold the plastic connector (not the wires) when unplugging to avoid damaging the cable. |
| PCIe 6/8-pin Power Connector | Provides extra power to dedicated graphics cards (GPUs). | Ensure the connector is fully seated and the locking clip clicks into place before turning on the computer. |

TASK-4

| Step | Component | SMPS Cable / Connector Used | Purpose |
| --- | --- | --- | --- |
| 1 | Install the SMPS in the PC case | — | Secure the SMPS with screws before connecting any cables. |
| 2 | Motherboard | 24-pin ATX Power Connector | Supplies the main power to the motherboard. |
| 3 | CPU | 8-pin EPS CPU Power Connector (4+4 pin) | Provides power to the CPU through the motherboard. |
| 4 | Graphics Card (GPU) | PCIe 6-pin, 8-pin, or 6+2-pin Connector | Supplies additional power required by the graphics card. |
| 5 | SATA SSD / HDD | SATA Power Connector | Powers SATA storage devices such as SSDs and HDDs. |
| 6 | SATA DVD Drive (if installed) | SATA Power Connector | Powers the optical drive. |
| 7 | Case Fans / RGB Hub (if required) | SATA Power or Molex Connector | Powers fans, RGB controllers, or accessories. |
| 8 | Final Check | All connected cables | Ensure every connector is fully inserted and the locking clips are engaged before powering on the PC. |